Data Sheet

Product: SHC-RCXXX

AV Cables

Smart Harmony offers high-quality RCA interconnect cables pre-terminated in commonly used lengths.

Cable Length::

Cable Length: 1.0M, 1.5M, 2.0M, 3.0M, 5.0M, 10M, 12M, 15M and 18M

Specifications:

| ITEM |  | DESCRIPTION |
| --- | --- | --- |
| Conductor | AWG | 24 |
|  | Material | Tinned Copper, Oxide-free |
|  | Size | 7/0.20mm±0.008mm |
| Insulation | Material | FM-PE+SKIN |
|  | Outer Diameter | 2.8±0.05mm |
| Spiral Copper | Material | Tinned Copper |
|  | Coverage | 90% |
| Jacket | Material | PVC |
|  | Color | Black |
|  | Outer Diameter | 4.0±0.15mm |

| ELECTRICAL CHARACTERISITICS | PHYSICAL PROPERTIES |
| --- | --- |
| - Rating: Temp 80℃ at 30V - Conductor Resistance: at 20℃ Max 24AWG: 93.25Ω/km - Insulation Resistance: 10mΩ/km min at 20℃, DC 500V - Dielectric Strength: AC 500/1 min - Coaxial Cable Impedance: 75±5Ωat 10MHz | - Tensile Strength: Unaged: 1500psi min, Aged: 70% min - Elongation: Unaged: 100%, Aged: 65% - Heat Shock Test: no cracking - Cold Bend Test: no cracking - Deformation Test: Max 50% - Flame Test: Passed UL VW-1 |